1972 CONVENTION ON THE INTERNATIONAL REGULATIONS FOR PREVENTING COLLISIONS AT SEA

Signed in London, United Kingdom on 20 October 1972

Rule 10 Traffic separation schemes

(a) This Rule applies to traffic separation schemes adopted by the Organization.

(b) A vessel using a traffic separation scheme shall:

(i) proceed in the appropriate traffic lane in the general direction of traffic flow for that lane;

(ii) so far as practicable keep clear of a traffic separation line or separation zone;

(iii) normally join or leave a traffic lane at the termination of the lane, but when joining or leaving

from  the  side  shall  do  so  at  as  small  an  angle  to  the  general  direction  of  traffic  flow  as

practicable.

(c) A vessel shall so far as practicable avoid crossing traffic lanes, but if obliged to do so shall cross as

nearly as practicable at right angles to the general direction of traffic flow.

(d)  Inshore  traffic  zones  shall  not  normally  be  used  by  through  traffic  which  can  safely  use  the

appropriate traffic lane within the adjacent traffic separation scheme.

(e)  A  vessel,  other  than  a  crossing  vessel,  shall  not  normally  enter  a  separation  zone  or  cross  a

separation line except:

(i) in cases of emergency to avoid immediate danger;

(ii) to engage in fishing within a separation zone.

(f)  A  vessel  navigating  in  areas  near  the  terminations  of  traffic  separation  schemes  shall  do  so  with

particular caution.

(g) A vessel shall so far as practicable avoid anchoring in a traffic separation scheme or in areas near

its terminations.

(h) A vessel not using a traffic separation scheme shall avoid it by as wide a margin as is practicable.

1972 CONVENTION ON THE INTERNATIONAL REGULATIONS FOR PREVENTING COLLISIONS AT SEA

UNOFFICIAL TEXT · CENTRE FOR INTERNATIONAL LAW · www.cil.nus.edu.sg                                 Page 12 of 34

(i) A vessel engaged in fishing shall not impede the passage of any vessel following a traffic lane.

(j) A vessel of less than 20 metres in length or a sailing vessel shall not impede the safe passage of a

power-driven vessel following a traffic lane.